THE D64CABLE

------------

This cable is specially designed for use with the Fcopy-PC program.

The cable will connect both C64 joystick ports to the PC's parallel

port.

You need a 25 pins connector, male, for the PC:

\ ------------------------------------------------------/

\  13  12  11  10  09  08  07  06  05  04  03  02  01 /

\                                                   /

\  25  24  23  22  21  20  19  18  17  16  15  14 /

\-----------------------------------------------/

You also need _two_ 9-pins c64 joy connectors, female:

\------------------------/

\  01  02  03  04  05  /

\                    /

\  06  07  08  09  /

\----------------/

D25-pin Connector                 Joyport       D9-pin Connector

-=-=-=-=-=-=-=-=-                 -=-=-=-       -=-=-=-=-=-=-=-=

2 <---------------------------1------------X---> 1 (JOYA0)

3 <---------------------------1------------X---> 2 (JOYA1)

4 <---------------------------1------------X---> 3 (JOYA2)

5 <---------------------------1------------X---> 4 (JOYA3)

6 <---------------------------1------------X---> 6 (BUTTON A/LP)

10 <---------------------------2----------------> 4 (JOYB3)

11 <---------------------------2----------------> 6 (BUTTON B)

12 <---------------------------2----------------> 3 (JOYB2)

13 <---------------------------2----------------> 2 (JOYB1)

15 <---------------------------2----------------> 1 (JOYB0)

25 <--------------------------1&2---------------> 8 (GND)

The 'X' marks a 1K resistor! These are essential to prevent the PC from

blowing up your C64 output!

Below is a new version of the D64 Cable that I have come up with so far. As you can see I have not perfected any speed improvements yet, But have simply added the wonderful little Activity light , like a 1541 has. I did this because I experienced that my 1084s monitor was giving off such a strong magnetic field that it would cause my PC monitor to develop shimmering & distortion.

The C64 Monitor does not need to be on, as the C64 & 1541 act as a slave to the PC, so I decided to turn off my 1084’s Monitor and add this Activity light to show when data is being sent or received.

I often send disks as a background task, while I am on the Internet, So if the C64 monitor is off, this Activity LED is the only indicator that the utility hasn’t crashed. I call this D64-Cable V2.0

THE D64CABLE  V2.0

-------------------

D25-pin Connector          Joyport       D9-pin Connector

-=-=-=-=-=-=-=-=-          -=-=-=-       -=-=-=-=-=-=-=-=

1----------------> 7 (+5v)

# Q1

2 <-------------------1------------X---> 1 (JOYA0)

25 <------------------1&2---------------> 8 (GND)

3 <-------------------1------------X---> 2 (JOYA1)

4 <-------------------1------------X---> 3 (JOYA2)

LED

5 <-------------------1------------X---> 4 (JOYA3)

6 <-------------------1------------X---> 6 (BUTTON A/LP)

10 <-------------------2----------------> 4 (JOYB3)

11 <-------------------2----------------> 6 (BUTTON B)

12 <-------------------2----------------> 3 (JOYB2)

13 <-------------------2----------------> 2 (JOYB1)

15 <-------------------2----------------> 1 (JOYB0)

Additional circuit that can be added to JOYPORT-1 to give an ACTIVITY LED, 																	                                                                                        Q1			                                                                          ( +5V )  7 ----------Z-----------------           E

( JOYA0 )  1 ----------Y-----------------           B

( GND )  8 ----------L-----------------           C

The ‘Z‘ marks a 390 ohms resistor!

The ‘Y‘ marks a 680 ohms resistor!

The ‘L‘ marks a GREEN Light Emitting Dioide.

The ‘Q1’ marks a BC557 PNP Transistor.

E = Emitter. , B = base. , C = Collector.