D64 CABLE & FCOPY-PC

--------------------

Fcopy-PC is designed for fast sending-receiving complete disk-sides from/to

a Commodore-C64 computer.

What you need:

- PC

- C64

- 1541 Diskdrive (or a newer drive with old Bios-kernal & mods )

- C64-UserPort <-> 1541 Cable          ]  BURST NIBBLER CABLE

- C64-Joyports <-> PC-Parallel Cable   ]  D64 CABLE

A complete disk-side will be sent from/to the C64 in 51 seconds,

with checksums, error-detection, sector- and ID header.

Fcopy uses a handshake protocol which guarantees that every disk can be

copied (with errors and special codes) to the PC, and back to the C64 also.

The Mhz and Clock speed of your PC does not matter at all, because of the handshake-protocol.

Where the regular D64 fails, Fcopy will work! Because Fcopy will also save

the format-ID and the checksums.

When copying a disk with Fcopy to a PC, two files will be created:

- A .D64 file, which can be used in most of the C64-emulators

- A .F64 file, which contains all the special codes.

The files in this archive:

FCOPYPC.CPP         The Object Oriented C source.

FCOPY.D64           Disk-image file of the Fcopy program for the C64.

D64CABLE-FCOPY.DOC  This 'doc' file.

FCCABLE.DOC         The Burst cable diagram.

FCOPYC64.EXE        The Fcopy program for the PC.

FCOPY.F64           Belongs to the FCOPY.D64 file, checksums & Header/Sector-ID.

FCOPYPC.PRG         Program file for the C64.

I did not design this cable or its software, but I can install and setup this system for anyone who wants it.

I have provided the diagram for the BURST NIBBLER CABLE, as this is easy to build. But take extreme care when installing in your drive that all pins are very tight in the IC socket. It will not cause any damage if it is simply loose.

I can Build, Test & Guarantee a BURST NIBBLER CABLE for anyone who wants one,

(5 year guarantee, comes with all software required ) for $35

I can Build, Test & Guarantee a D64-CABLE for anyone who wants one,

(5 year guarantee, comes with all software required ) for $34

---------------------------------------------------------------

If someone wants me to build the lot, Total cost =  $69 (Australian)

$39 (USA)

The D64 cable, on the other hand, is a different creature altogether.

The D64 cable contains electronic components and requires some skill & care to construct. It can easily blow up the C64 if not assembled correctly because the voltages from the PC parallel port are much higher than what the C64 Joyports can handle, so any small short can kill your hardware.

I will not be held responsible or liable in any way for any cables that I did not build 100% my self. If you build your own cable and make a mistake, so be it on your head & your head alone.

I will also take no responsibility for peoples misinterpretations.

If you still believe you have the electronic knowledge and skill to build the D64 cable, I can send a diagram for it on request.

People Highly Experienced in electronics will find the D64 Cable quite simple.

THE VORTEX                                                             5-12-2000